Electric Drives

and Controls

Linear Motion and

Assembly  Technologies

Hydraulics

Pneumatics

Service

R911328248

01

Bosch Rexroth AG

Electric Drives and Controls

P.O.Box 13 57

Rexroth IndraMotion MTX micro

97803 Lohr, Germany

Bgm.-Dr.-Nebel-Str. 2

97816 Lohr, Germany

Phone

Fax

+49 (0)93 52-40-0

+49 (0)93 52-40-48 85

www.boschrexroth.com

Printed in Germany

DOK-MTXMIC-EASY****V10-CO01-ZH-P

R911328248

Bosch Rexroth AG  |  Electric Drives

and Controls

力士乐  | 简明调试手册

题目

力士乐

Rexroth IndraMotion MTX micro

文件类型

文件类型代号

内部存档附注

简明调试手册

DOK-MTXMIC-EASY****V10-CO01-ZH-P

RS-5be588e855ee71b50a6846a000fccaae-1-zh-CN-3

文件用途     简明调试手册 IndraMotion MTX micro

更改过程

出版

颁发日期

04/2009

备注

120-1450-B302/ZH -01

Erstausgabe

版权     © Bosch Rexroth AG, 2009

没有明示授权，不得复制本文档、将其转交给其它方，以及使用或传达其中的

内容。违反者将承担损失赔偿。违反者将承担损失赔偿。在实用新型或设计的

专利授予或注册授予中保留所有权利 (DIN 34-1)。

真实性

规格数据仅用于产品说明，且不被视为某种担保，除非在合同中有明示的确

认。保留关于本文档内容和本产品可用性的所有权利。

出版方     Bosch Rexroth AG

Bgm.-Dr.-Nebel-Str. 2 ■ D-97816 Lohr a. Main

电话：+49 (0)93 52/ 40-0 ■  传真：+49 (0)93 52/ 40-48 85

http://www.boschrexroth.com/

Dept. BRC/SIM (JeNo, FrSc)

注意

本文档使用无氯漂白纸印刷。

简明调试手册  | 力士乐

Electric Drives  |  Bosch Rexroth AG

and Controls

I/IV

目录

目录

页数

1

系统总览......................................................................................................................... 5

简介......................................................................................................................................................... 5

控制系统的性能....................................................................................................................................... 6

4 轴配置 - 针对典型铣床.........................................................................................................................  7

3 轴配置 – 针对典型车床......................................................................................................................... 8

1.1

1.2

1.3

1.4

2

2.1

2.2

2.2.1

2.2.2

2.3

硬件选择和连接.............................................................................................................. 9

HCT02/HCQ02.......................................................................................................................................  9

电机....................................................................................................................................................... 12

QSK................................................................................................................................................... 12

MAD................................................................................................................................................... 13

Anschlusskabel..................................................................................................................................... 14

HMI 操作面板........................................................................................................................................ 15

VDP80.1FBN-C1-NN-EN................................................................................................................... 15

VDP80.1FAN-C1-NN-EN................................................................................................................... 16

硬件的选择和连接................................................................................................................................. 17

1.1 设计说明......................................................................................................................................  17

电源................................................................................................................................................  17

接地设计.........................................................................................................................................  17

外部电阻.........................................................................................................................................  18

24V 控制电压供给........................................................................................................................... 19

急停和电力接触器的布线................................................................................................................ 20

MAD 主轴电机的接口.....................................................................................................................  21

标准异步电机的接口.......................................................................................................................  22

一个可选的编码器的接口 (X8)........................................................................................................  23

伺服电机的接口..............................................................................................................................  24

外部刹车制动器的控制.................................................................................................................... 25

数字输入信号和数字输出信号......................................................................................................... 26

配电柜的构造.................................................................................................................................. 27

2.4

2.4.1

2.4.2

2.5

2.5.1

3

3.1

3.1.1

基础工程应用 / 安装.....................................................................................................  29

安装 IW Engineering............................................................................................................................. 29

必要条件............................................................................................................................................  29

安装 Rexroth IndraMotion MTXmicro 的硬件必要条件...................................................................  29

安装 Rexroth IndraMotion MTXmicro 的软件必要条件...................................................................  29

安装程序............................................................................................................................................  29

恢复基础工程........................................................................................................................................  32

建立通讯...............................................................................................................................................  34

下载 NC 参数........................................................................................................................................  35

3.1.2

3.2

3.3

3.4

4

4.1

4.2

PLC 调试...................................................................................................................... 37

程序编译...............................................................................................................................................  37

系统程序...............................................................................................................................................  38

II/IV

Bosch Rexroth AG  |  Electric Drives

and Controls

力士乐  | 简明调试手册

目录

页数

4.2.1

4.2.2

4.2.3

4.3

4.3.1

4.3.2

4.3.3

4.3.4

4.3.5

4.3.6

4.4

4.4.1

4.4.2

4.4.3

4.4.4

4.4.5

4.4.6

应用程序的组成部分..........................................................................................................................  38

主程序 prMAIN................................................................................................................................... 39

主程序的编程..................................................................................................................................... 40

辅助功能...............................................................................................................................................  40

基础...................................................................................................................................................  40

辅助功能类型..................................................................................................................................... 40

确认方式............................................................................................................................................  41

辅助功能的编程.................................................................................................................................  42

辅助功能组......................................................................................................................................... 43

特殊功能............................................................................................................................................  45

诊断 (机床状态指示).............................................................................................................................. 46

基础...................................................................................................................................................  46

MZA 信息的显示................................................................................................................................ 46

信息等级............................................................................................................................................  47

信息的编程......................................................................................................................................... 48

信息文本的管理.................................................................................................................................  48

信息文本的输入.................................................................................................................................  49

5

5.1

5.2

5.2.1

5.2.2

5.2.3

5.2.4

5.2.5

5.3

设定 NC 参数................................................................................................................ 51

引言....................................................................................................................................................... 51

NC 配置................................................................................................................................................  52

引言...................................................................................................................................................  52

基本设计............................................................................................................................................  53

参数组................................................................................................................................................ 54

快速编辑器......................................................................................................................................... 55

参数说明/ 帮助................................................................................................................................... 56

基本配置...............................................................................................................................................  57

引言...................................................................................................................................................  57

驱动器上轴的分配.............................................................................................................................. 59

5.3.1

5.3.2

6

6.1

6.2

6.2.1

6.2.2

6.3

6.3.1

6.3.2

安装驱动器................................................................................................................... 67

搜索驱动器............................................................................................................................................ 67

安装驱动参数........................................................................................................................................  68

下载基本参数..................................................................................................................................... 68

输入制动电阻器电阻值....................................................................................................................... 70

驱动优化器............................................................................................................................................ 71

基础...................................................................................................................................................  71

优化速度控制环.................................................................................................................................  72

共振的确定和带宽的检测................................................................................................................ 72

设置参数 Kp 和 Tn.......................................................................................................................... 75

记录速度控制环的阶跃相应............................................................................................................  76

优化位置控制环.................................................................................................................................  78

基础................................................................................................................................................  78

设置参数 Kv.................................................................................................................................... 78

设置加速度预控制........................................................................................................................... 79

6.3.3

简明调试手册  | 力士乐

Electric Drives  |  Bosch Rexroth AG

and Controls

III/IV

目录

页数

设置 Jerk......................................................................................................................................... 80

7

IndraMotion MTX micro 操作界面简介......................................................................... 83

面板硬件 VDP80................................................................................................................................... 83

用户界面的基本布局.............................................................................................................................  85

对表格的操作........................................................................................................................................  86

文本编辑...............................................................................................................................................  87

操作模式...............................................................................................................................................  87

操作区域...............................................................................................................................................  88

PLC 和 HMI 软件的接口........................................................................................................................ 93

系统的安装和升级................................................................................................................................. 93

7.1

7.2

7.3

7.4

7.5

7.6

7.7

7.8

8

数据备份，数据恢复和系统克隆................................................................................... 95

9

9.1

9.2

9.2.1

9.2.2

附录.............................................................................................................................. 97

接线图................................................................................................................................................... 97

参数列表...............................................................................................................................................  99

NC 配置.............................................................................................................................................  99

PLC-CNC 接口................................................................................................................................. 101

起始地址.......................................................................................................................................  101

主板 Input / Output........................................................................................................................ 102

辅助功能.......................................................................................................................................  103

10

服务和支持 ................................................................................................................ 105

索引............................................................................................................................ 107